REVIEW

by Prof. Dr. Eng. Pavel Evstatiev Pavlov

Department of Underground Construction

Faculty of Mining Technology

at the University of Mining and Geology "St. Ivan Rilski"

Sofia

of a PhD dissertation:

"Investigation of the Stress and Strain State of Tunnel Structures during Construction with Tunnel Boring Machines in the Complex Engineering Conditions of the Sofia Metro"

The dissertation is presented by MSc. Eng. Vladimir Rumenov Penev,

PhD student at the Department of Underground Construction, Faculty of Mining Technology, University of Mining and Geology "St. Ivan Rilski", Sofia

For the acquisition of the scientific and educational degree "Doctor" in the professional field 5.7. "Architecture, Construction and Geodesy", scientific specialty "Underground Construction"

Scientific supervisor of the PhD student: Assoc. Prof. Dr. Eng. Ivan Stoyanov Mitev

The review was prepared in accordance with Order RD -13-05 of 13.02.2026 and is based on §2 (4) of Appendix 4 of the "Rules and Procedures for Admission and Training of Doctoral Students and Acquisition of the Scientific and Educational Degree "Doctor" and the Scientific Degree "Doctor of Sciences" at the University of Mining and Geology "St. Ivan Rilski"", in accordance with the decision of the Faculty Council of the Faculty of Mining Technology of 29.01.2026. /Minutes No. 6, academic year 2025/2026/, as well as in accordance with the requirements of the Law on the Development of the Academic Staff in the Republic of Bulgaria and the Regulations for its Application.

Biographical data

Engineer Vladimir Rumenov Penev was born in the city of Silistra. He completed his secondary education at the "Peyo Yavorov" Language High School in 2008. Then he became a student in the "Construction of Buildings and Facilities" course of studies at the University of Architecture, Civil Engineering and Geodesy, Sofia. He graduated in 2013 with an average grade of Excellent 5.56. Subsequently, he acquired a MSc degree in "Underground Construction" at the University of Mining and Geology "St. Ivan Rilski", Sofia, with an average grade of Excellent 5.94.

From 2013 to 2019, Eng. Penev worked as a design engineer in the following companies: "DE Studio"; "Angel Project" EOOD; "PGM Engineering" EOOD; "Steruni" EOOD.

In 2019 Eng. Vladimir Penev applied and was successfully enrolled as a full-time doctoral student at the Department of Underground Construction. In 2020, he became an Assistant Professor at the Department of Underground Construction. Currently, Ass. Prof. Penev teaches the following disciplines in the Bachelor's degree programme - "Ground Mechanics and Foundations" - seminars and course project; "Underground Urban Infrastructure" - seminars and course project; "Construction Mechanics of Underground Facilities" - seminars. In addition, Ass. Prof. Penev teaches the following disciplines for the Master's degree programme: "Dynamics and Stability of Underground Facilities. Seismics" - seminars for the "Underground Construction" course of studies; "Seismic Processes" - seminars for the "Geotechnics" course of studies; "Geotechnical Design" - seminars for the "Geotechnics" course of studies.

Ass. Prof. Penev is a member of the following scientific organisations: Chamber of Engineers in Investment Design, Bulgaria; Scientific and Technical Union of Mining, Geology and Metallurgy; International Society of Rock Mechanics and Rock Engineering.

He is fluent in English and uses German.

Description and analysis of the materials submitted for review

The PhD student Eng. Vladimir Penev has submitted the following materials:

• Dissertation on the topic "Investigation of the stress and strain state of tunnel structures during construction with tunnel boring machines in the complex engineering conditions of the Sofia metro". The PhD thesis is elaborated on 172 pages, including 9 chapters for presenting the reference review, the theoretical basis and solving the main formulated tasks, a list of the main contributions and a list of the reference literature used. A total of 104 reference sources is cited, of which 77 are in Latin and 26 in Cyrillic alphabet. The remaining cited reference sources are Internet addresses (websites). There are 82 figures and 28 tables in the PhD thesis and their numbers correspond to those in the dissertation.

Relevance of the issue and motivation of the research

The topic of the dissertation is determined by the need to expand the Sofia metro as a factor for sustainable transport and for reducing traffic and pollution in the capital. The focus is on construction activities in a densely built-up urban environment, where the interaction "soil - structure - technology" and deformations management are critical. As an example, the divergence/intersection of Metro Line 3 under Metro Line 1 in the area under Knyazheska Gradina, in the interval between km 6+380 and 6+680 for Line 3 and between km 6+600 and 6+550 for Line 1, with a vertical distance of only 140 cm, is analysed. The task is typical of complex engineering conditions due to the operation of the tunnels of Metro Line 1, the strict limitations on permissible deformations, the maintenance of the technological parameters of the tunnel boring machine (in this case EPB) within narrow limits and the heterogeneous geological and hydrogeological conditions in the Sofia basin. At the same time, the reliability of the predictions depends on the quality of the data, the chosen constitutive model and the verification and validation procedures. The research is up-to-date and aimed at scientifically based prediction and management of deformations in mechanized tunnel construction in an urban area.

Structurally, the PhD dissertation is divided as follows:

- Chapter One

The first chapter contains an introductory part. It clearly and specifically defines the objectives and tasks of the PhD thesis.

- Chapter Two

The second chapter includes an overview. A comparative analysis of the specific geological conditions of the Sofia Metropolitan with the circumstances in the construction of other similar underground facilities worldwide has been made. Special attention has been paid to the prerequisites and conditions for the application of a tunnel boring machine. The fastening methods, the benefits of injection and the probable causes of subsidence have been examined. The numerical models for calculating subsidence have been described. The seismic impacts on the underground structure have been examined. Attention has been paid to the current regulatory framework in the Republic of Bulgaria, the European standards and Eurocodes.

- Chapter Three

The chapter is extremely detailed and concerns the determination of the soil/earth backpressure during the construction of a tunnel section of the third Metro diameter of the Sofia Metropolitan. The route is accurately described. A specific lot of the third Metro diameter has been examined - between Metro Station No. 8 and Metro Station No. 9. The geological conditions have been analysed. The concept of the Earth Pressure Balance (EPB) pressure has been developed during the operation of the TBM (Tunnel Boring Machine) in the specific geological conditions. Comparisons of EPB pressure have been made using different methods. The comparisons are visualised in graphical and tabular form. Monitoring has been developed. The calibration of the subsidence curve is described, based on the monitoring data. Updated EPB pressure values for the specific section are proposed.

- Chapter Four

The chapter is dedicated to the possibilities and benefits of numerical modelling when using TBM. Models of the earth-rock mass and its interaction with various structural materials have been made. The emphasis falls on modelling the segmental lining of the metro tunnel. Basic dependencies have been derived and an assessment of the uncertainties has been formulated, related to the current Eurocodes and regulatory framework.

- Chapter Five

The chapter is of considerable size and examines the intersection of two Metro diameters - M1 and M3 of the Sofia Metropolitan. The applicability of the EPB technology has been studied, taking into consideration the balance of the soil pressure at the intersection of the two metro routes. Based on the specific engineering-geological and hydrogeological conditions, a safety factor has been derived. The structural elements /segments/ of the primary lining of line 1 and line 3 have been dimensioned. The movement of the tunnel boring machine /TBM/ has been modelled, the aim being to select several characteristic zones: Unexcavated section; Excavated section with applied frontal pressure in front of the cutter head; Supported section with two-dimensional elements, with reduced rigidity, used for controlled adaptation of the soil deformations to the conicity of the TBM shield; Supported section with two-dimensional elements modelling the final tunnel lining - the tubbing rings.

The conicity of the shield has been modelled using two-dimensional finite elements with a reduced modulus of elasticity, which allow deformations to be predicted. The displacements in the vault have been calculated at a frontal backpressure in the face of Р=150 kPa. Variants with backpressure of Р=100 kPa and Р=250 kPa have also been considered. A variant with a pipe umbrella, which is installed in the intersection zone of the two tunnels, has been analysed. Conclusions have been made and the absolute deformations have been derived. A final comparison of the load states’ models with the three selected backpressure values has been made. The main conclusion is that the reduction in vertical displacements is strongly pronounced at an extreme increase in the frontal pressure. The pipe umbrella has a positive influence. It can be perceived as an additional protection in unforeseen situations and can be built without the risk of forming a trough on the ground surface.

- Chapter Six

The conclusions drawn in this section are relevant and have a clearly expressed practical focus. A review of the contemporary regulatory framework has been carried out. A specific section of the third metro diameter has been examined. Specific values of the backpressure according to different methodologies have been emphasized - "Singapore", "Empirical method", etc. The real monitoring system at the divergence of Metro diameters 1 and 3 has been analysed. Six numerical models have been created, depending on whether additional measures are included to strengthen the soil when the tunnel from Metro diameter 3 passes under the existing tunnel tubes of Metro diameter 1. Proposals for additional strengthening measures concerning the stress-strain state and the expected displacements have been given.

- Chapter Seven

The doctoral candidate has summarized the main results. They basically repeat the written conclusions. In the process of TBM operation, the backpressure of the face chamber should be calibrated based on monitoring data, with continuous monitoring of the subsidence of the ground surface and the deformations of the adjacent urban infrastructure. It is recommended that when entering the metro stations areas, the EPB pressure should be reduced to zero values. In these areas, the stability of the face is ensured by preliminary strengthening of the soil body or by additional structural solutions. Similarly, when the TBM leaves, the backpressure is smoothly increased until reaching a strong zone and until the initial rings of the lining are installed. In critical areas, at a shallow depth or passing under existing buildings and facilities, the backpressure is selected extremely accurately, by carefully monitoring the dynamics of the piezometric level and constant monitoring of groundwater.

In these areas, more intensive monitoring is carried out. In the computational models, lower safety factors are assumed in order to minimize the impact zone around the tunnel tube.

The comparison between the results of the numerical models and the values obtained by analytical formulas shows significant similarity in terms of the determined values of backpressure in the face. The possibility of installing a pipe umbrella in the area of the intersection of two metro tunnels has been considered. Such a measure is reflected in a reduction of about 10% of the expected deformations. The construction of a pipe umbrella can be considered as a preventive and recommended measure for critical sections.

- Chapter Eight

In this chapter, the doctoral student has derived the main scientific and research-applied contributions. They are well systematized and correspond to the conducted research and the content of the PhD dissertation. The main ones are:

- Summarised comparative tables have been developed that systematise the requirements of various regulatory documents and approaches, providing a practical tool for analysis and selection of design solutions in tunnel construction.

- An algorithm for determining soil backpressure when using EPB tunnel boring machines has been developed, which integrates analytical, empirical and normative approaches. The algorithm provides a consistent framework for assessing and checking the frontal pressure under various geotechnical conditions.

- Procedures for verification, validation and assessment of the indeterminacies of numerical models in the context of geotechnical tasks have been derived. The main sources of indeterminacies arising from the geological environment, model assumptions and input parameters have been considered.

- A methodology has been formulated for modelling the conicity of the TBM shield in the most unfavourable expected conditions, characterised by volume losses VL = 0.65%. The methodology improves the assessment of deformations in the area around the tunnel.

- An algorithm has been developed for numerical modelling  of the process of driving the tunnels with EPB tunnel boring machines, which encompasses the main excavation stages, the setting of face backpressure and the interaction between the soil mass and the structural elements.

- An algorithm has been developed (using the finite element method) for the numerical modelling of tunnel excavation processes carried out with Earth Pressure Balance (EPB) tunnel boring machines. The model incorporates the principal stages of excavation, defining the face backpressure, and the interaction between the surrounding soil mass and the structural elements involved in the tunnelling process.

- Chapter Nine

The doctoral candidate has carefully considered current trends and emerging directions in future research. In this context, updated and comprehensive example calculations, fully aligned with the new generation of Eurocodes, are planned for development in the field of underground and mining construction. These elaborated and systematically justified examples are intended to enhance the understanding of reliability concepts, modelling approaches, and the assessment of uncertainties in the design of underground structures.

Another important area is the expansion of the applicability of the algorithms developed within the dissertation. The algorithms created for determining the EPB backpressure and for numerical modelling can be applied to other types of facilities, such as road and railway tunnels, underground drifts and mine workings. The interaction between adjacent tunnels and other underground facilities during seismic events, as well as the influence of this interaction on the stress-strain state and the operational reliability of the facilities is also of interest.

Note

A limitation of the study is that the obtained dataset and the proposed EPB backpressure values have not been validated against in situ geodetic and mine surveying measurements. Furthermore, actual surface settlement troughs have not been reproduced or visualized.

Conclusion

The documents and materials presented are in accordance with the requirements of the Law on the Development of the Academic Staff in the Republic of Bulgaria and the Regulations for its Application.

The doctoral student has developed the PhD dissertation comprehensively and in great detail. He demonstrates good mathematical knowledge and successfully uses specialised software products. He is capable of developing, testing, modifying, and approximating algorithms for practical applications.

Despite the aforementioned remark, and based on the submitted materials and my review, I provide a positive assessment of the dissertation "Investigation of the stress and strain state of tunnel structures during construction with tunnel boring machines in the complex engineering conditions of the Sofia metro", presented by MSc. Eng. Vladimir Rumenov Penev. I propose to the esteemed Scientific Jury to confer upon MSc. Eng. Vladimir Rumenov Penev the educational and scientific degree "Doctor" in the scientific specialty "Underground Construction", professional field 5.7. "Architecture, Construction and Geodesy".

19.02.2026                                           Prepared by:

Sofia                             		/ Prof. Dr. Eng. Pavel Evstatiev Pavlov /