REVIEW

By:

Prof. Dr. Eng. Ivaylo Georgiev Koprev,

University of Mining and Geology “St. Ivan Rilski”

Regarding:

Doctoral dissertation for obtaining the educational and scientific degree “Doctor” in professional field 5.8 “Prospecting, Extraction and Processing of Mineral Resources”, scientific specialty “Mechanization of Mines”.

Basis for presenting the review:

Participation in the Scientific Jury approved under Order No. RD13-23/22.05.2026 of the Rector of the University of Mining and Geology “St. Ivan Rilski”.

Author of the dissertation: Eng. Stanimir Georgiev Stanchev

Title of the dissertation:

“Investigation of the Mechanical and Technological Parameters of a Rotary Crusher – Eccentric Type”

1. Information about the PhD Candidate

The PhD candidate, Eng. Stanimir Georgiev Stanchev, graduated from the University of Mining and Geology “St. Ivan Rilski”, where he consecutively obtained the educational qualification degree Bachelor in Mechanization of Mining Production and the educational qualification degree Master in Mechanization for Mineral Processing. Since 2022 he has been a PhD student in the doctoral program “Mechanization of Mines” within professional field 5.8 “Prospecting, Extraction and Processing of Mineral Resources”.

Since September 2024 he has been an assistant lecturer at the University of Mining and Geology “St. Ivan Rilski”, combining teaching activity with research work in the field of mining mechanization and machines for processing mineral raw materials. The PhD candidate possesses solid engineering preparation and significant practical experience in machines and technological systems. He is proficient in specialized engineering software, including AutoCAD Mechanical, used for technical design and three-dimensional modeling.

He has successfully passed all examinations included in his individual study plan, according to Certificate SSPM No. 385-2026 from 12.05.2026. He has been discharged with the right to defend under Rector’s Order No. UD-16-62/20.05.2026. The dissertation was presented and discussed at a meeting of the Department Council of “Mechanization of Mines”, Mining-Electromechanical Faculty, held on 11.05.2026.

The set of materials submitted by Eng. Stanimir Stanchev on electronic media complies with the Rules and Procedures for Admission and Training of PhD Students and for Obtaining the Educational and Scientific Degree “Doctor” at the University of Mining and Geology “St. Ivan Rilski”.

2. General Characteristics of the Dissertation and Evaluation of the Results

The dissertation is developed in a volume of 213 pages and includes eight main sections: introduction, literature review, theoretical framework, presentation of the conducted studies, conclusion, scientific-applied and applied contributions, publications, references, and appendix. It cites 120 literary sources (33 in Latin script and 87 in Cyrillic) and includes 178 figures, 60 graphs, and 40 tables.

The author demonstrates in-depth knowledge of the researched problem, enabling a scientifically grounded applied approach to solving the assigned tasks.

The aim of the dissertation is to investigate the relationships between the main parameters of the eccentric-type rotary crusher and its energy efficiency, productivity, and quality of the crushed product. A combined approach is applied, including theoretical modeling, 3D CAD modeling, FEM/CAE simulations, experimental studies, and statistical processing of the results.

Tasks required to achieve the aim:

– Analysis of the current state of crushing machines and clarification of the place of the eccentric-type rotary crusher in classification schemes. – Justification of the structural and technological significance of the eccentric-type rotary crusher as a machine with intermediate characteristics between known crushing solutions. – Development of a theoretical framework for determining the main kinematic, mechanical, and technological parameters. – Creation of a 3D CAD model of the crusher as a basis for geometric, strength, and simulation studies. – Preparation of a FEM/CAE assessment of the stress-strain state of the main elements under specified operating loads. – Development of an experimental setup and methodology for studying the working process under controlled values of the main factors. – Experimental studies with limestone and diabase at different values of frequency, discharge gap, and number of chambers. – Grain-size analysis of the final product and determination of the influence of the operating regime on mean particle size and fine fractions. – Statistical processing of the results and evaluation of regression models by R², adjusted R², P-value, and F-criterion. – Multi-objective optimization for determining rational compromise operating regimes.

A complex approach is adopted, including analysis and summarization of scientific and practical results from previous studies, statistical and analytical processing of data in mathematical modeling.

The dissertation contains eight chapters presenting the research work.

Summary of Chapters

Chapter I – Literature Review

The literature review shows that eccentric-type rotary crushers are gaining a stable place among modern crushing machines. Analysis of published developments, structural solutions, and application areas demonstrates that this type of crusher combines advantages of classical jaw and roll machines while enabling higher productivity. The review highlights the need for deeper examination of structural, kinematic, and operational specifics to fully evaluate their potential.

Chapter II – Aim and Tasks

Defines the aim and tasks of the dissertation.

Chapter III – Theoretical Determination of Parameters

Examines the structural design, operating principle, and kinematics of the crusher. Derives dependencies for productivity, power, and crushing forces. Presents 3D CAD modeling and FEM analysis for evaluating stresses, deformations, and modal characteristics. The chapter concludes with summarized findings on structural stability and rotor loading.

Chapter IV – Experimental Program

Presents the complete experimental program, including 40 main and additional tests. The aim is to establish quantitative relationships between rotational frequency, discharge gap, and number of chambers, and the key technological indicators: – product quality (Dₛᵣ), – productivity (Qₘ), – energy consumption (Pₛᵣ), – vibration behavior.

The chapter concludes with summarized findings identifying the most influential factors and favorable parameter combinations.

Chapter V – Statistical Analysis and Multi-Objective Optimization

Includes ANOVA, regression modeling, evaluation of R², adjusted R², F-criterion, P-value, and residual errors. The models show high adequacy and consistency with experimental data.

Key conclusions: – Statistical models reliably describe machine behavior. – Frequency is the strongest factor. – Discharge gap is a compromise parameter between quality and productivity. – Number of chambers affects stability and uniformity. – Multi-objective optimization identifies a rational operating region rather than a single optimum.

Chapter VI – Conclusions and Contributions

Scientific-Applied Contributions

– Classification of the eccentric-type rotary crusher as a distinct machine class. – Development of a consistent methodology combining theoretical, simulation, and experimental studies. – Construction of regression models describing the influence of frequency, gap, and number of chambers. – Development of multi-objective optimization integrating three key technological indicators. – Validation of theoretical models through comparison with experimental results.

Applied Contributions

– Determination of rational operating regimes balancing quality, productivity, and energy efficiency. – Creation of dependencies and recommendations for machine adjustment in real production conditions. – CAD/FEM model applicable for future design. – Experimental results supporting industrial implementation.

3. Evaluation of Contributions

The contributions correspond to the achievements of MEng. Stanimir Stanchev and expand existing scientific knowledge. The research is extensive, and the results are summarized into nine scientific-applied contributions.

4. Evaluation of Publications and Abstract

The two publications are directly related to the dissertation and reflect its main achievements. They show that the candidate is an analytical researcher combining strong theoretical preparation with practical experience.

The abstract accurately reflects the dissertation’s content, methods, and contributions. It meets all regulatory requirements.

The candidate fully meets the mandatory scientometric criteria for obtaining the degree Doctor.

5. Critical Remarks and Recommendations

– The experimental range of parameters is narrow; broader ranges would improve applicability to industrial machines.

– No direct comparison with industrial crushers is provided.

– Vibration analysis is used diagnostically; deeper dynamic analysis would be beneficial.

– No quantitative comparison with classical energy hypotheses (Rittinger, Kick, Bond).

– Some regression models could be presented more structurally (tables/graphs).

These remarks do not diminish the scientific value of the work.

6. Conclusion

After reviewing the dissertation and accompanying publications, and based on the analysis of their significance and contributions, I confirm that the dissertation and publications meet the requirements of the Law on the Development of Academic Staff in the Republic of Bulgaria, its regulations, and the institutional rules of the University of Mining and Geology “St. Ivan Rilski”.

I give a positive evaluation of the dissertation “Investigation of Mechanical and Technological Parameters of a Rotary Crusher – Eccentric Type” developed by Eng. Stanimir Stanchev and consider that he should be awarded the educational and scientific degree Doctor.

12 August 2026 Sofia

Reviewer: (Prof. Dr. Eng. Ivaylo Koprev)